| - PHASED ARRAY ULTRASONIC TESTING REPORT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - JOB DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Client: |  |  | - NCOC N.V. |  |  |  | - Project: |  |  |  | - ESKENE WEST KARABATAN |  |  |  |  |  |  |  |  |  | - Work Location: |  |  |  | - Samal Utility 2 |  |  |  |
| - JOB DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Brief Description of Job: |  |  |  |  |  |  | - Phased Array weld inspection of 4inch Elbow butt weld |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Line No.: |  |  |  |  |  |  | - D1-590-ZH-001B |  |  |  |  |  |  |  |  | - Location: |  |  |  |  | - Samal Utility 2 |  |  |  |  |  |  |  |
| - Material: |  |  |  |  |  |  | - Carbon steel |  |  |  |  |  |  |  |  | - Surface Condition: |  |  |  |  | - Smooth |  |  |  |  |  |  |  |
| - Nominal thickness |  |  |  |  |  |  | - 6.02 mm |  |  |  |  |  |  |  |  | - Diameter |  |  |  |  | - 4 inch |  |  |  |  |  |  |  |
| - Part temperature |  |  |  |  |  |  | - 30 °C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - INSPECTION PROCEDURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Procedure No: |  |  |  |  | - QP-11-PAUT-CM-Q01 REV 01 |  |  |  |  |  |  | - In accordance with: |  |  |  |  |  | - ASME SEC V Art 4 |  |  |  | - In accordance with: |  |  |  |  | - ASME B31.3 Para 344.6.2 |  |
| - INSPECTION EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| S. No | Equipment/ Material Name |  |  |  |  |  | Manufacturer |  |  |  |  |  |  | Serial No |  |  |  |  | Calibration certificate No |  |  |  |  | Calibration Expiry date |  |  |  |  |
|  | OmniScan MX2 |  |  |  |  |  | Olympus |  |  |  |  |  |  | 103625 |  |  |  |  | BK-01-0126 |  |  |  |  | 30.01.2019 |  |  |  |  |
|  | Navship Cal block |  |  |  |  |  | Olympus |  |  |  |  |  |  | 015716 |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| - EQIPMENT PARAMETERS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - CALIBRATION BLOCK DETAILS |  |  |  |  |  |  |  |
| - Mode |  |  |  | - N/A |  |  |  | Filter |  | 5MHz |  |  | - Points quant'y |  |  |  |  | - 640 |  |  | Cal block |  |  |  |  | Navship Cal block |  |  |
| - Frequency |  |  |  | - 5 MHz |  |  |  | Rectifier |  | FW |  |  | - Sum gain |  |  |  |  | - Auto |  |  | Material |  |  |  |  | CS |  |  |
| - Energy |  |  |  | - 40v |  |  |  | Video filter |  | Off |  |  | Ref sensitivity |  |  |  |  | 33.7dB |  |  | Thickness Range |  |  |  |  | 75mm |  |  |
| - Pulse width |  |  |  | - Auto |  |  |  | Averaging |  | 1 |  |  | Scan sensit'ty |  |  |  |  | 6 dB |  |  | temperature |  |  |  |  | Room temperature |  |  |
| - PRF |  |  |  | - Optimum |  |  |  | - Focus depth |  | - 20 |  |  |  |  |  |  |  |  |  |  | - Correction Factors |  |  |  |  | - N/A |  |  |
| - Probe |  |  |  | - Ambient |  |  |  | - Couplant |  | - Gel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - SCAN PLAN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Test Ref |  | - Scan type |  |  |  | - Beam type |  |  | - Index offset |  |  |  |  |  | - Start element |  | - Active elements |  |  | - Minimum angle |  |  | - Maximum - angle |  |  |  |  | - Angle Step |
|  |  | Sectorial |  |  |  | Angle |  |  | 90 |  |  |  |  |  | -7 |  | 49 |  |  | 16 |  |  | 40 |  |  |  |  | 70 |

CALIBRATION DETAILS

Calibration on Navships Block using 12.5mm depth hole: - Phased array weld inspection has been carried out on (ø4”X6.02mm) Elbow butt weld. SCANNED WELD SBW-01 SBW-02

SCAN PLAN

RESULTS Tag No; Joint No.; Indication No; Type of Indication; Location (mm); Length (mm); Depth (mm); Height (mm); Surface/ Sub surface Flaw; Results (A or R); Scanned length (mm). Start; End; Start; End. D1-590-ZH-001B; SBW-01; -; NSD; -; -; -; -; -; -; -; A; 0-360*. D1-590-ZH-001B; SBW-02; -; NSD; -; -; -; -; -; -; -; A; 0-360*. Note: - Results (A-Accept, R-Reject) - NSD : No Significant Defect - ID : Inner Diameter or Surface - OD : Outer Diameter or Surface - Datum ‘0’ of the weld has taken on top dead center and scanned in clock wise direction. SCAN IMAGE SBW-01 SBW-02 APPROVALS:. Operator UT Level III Cer.No. 282671; ARUL VIGNESH cer.exp date: 01.05.2021; Signature: ...........................; Date: 04.07. 2018. Supervisor UT Level III cer. No #309566; PRAGADA SANTHOSH KUMAR cer.exp date: 13.06.2022; Signature: ...........................; Date: 04.07. 2018. Client Representative:; :………………………….….; Signature: ...........................; Date: ............................