| ULTRASONIC TESTING REPORT |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Company/Client |  | NCOC N. V. |  | Project |  | ESKENE WEST |  | Location |  | Unit - 560 |
| Line No/Tag No: |  | According to table |  | Material |  | According to table |  | Día × Nominal Thickness |  | According to table |
| Procedure No: |  | QP-11-UTT-Q01 Rev 03 GOST 28702-90 |  | Calibration Block |  | Step wedge |  | Probe S/N |  | D7908 1214571 |
|  |  |  |  | Calibration block Range |  | 2.5-25.05 mm |  | Angle |  | 0º |
| Manufacturing Code |  | ASME Sec VIII D.1 ASME B31.3 |  | Calibration Temp |  | +10ºC |  | Frequency |  | 7.5 MHz |
|  |  |  |  | Object Temp |  | +12ºC |  | Size |  | 5.1 mm |
| Acceptance Standard: |  | Acc. to the client specif. |  | Correction factors |  | n/a |  | Crystal |  | Dual |
| Surface condition |  | Coated |  | Couplant |  | UT Gel |  | Accuracy |  | ± 0,1 mm |
| S. No | Equipment/ Material Name |  | Manufacturer |  | Serial No |  | Calibration certificate No |  | Calibration Expiry date |  |
|  | DL-38 |  | OLYMPUS |  | 171421601 |  | BK-01-0674 |  | 20-03-2020 |  |
|  | Step wedge calibration block |  | NDE-TECH |  | NDE-8363 |  | - |  | - |  |
| Job description: - UT thickness was performed on the Nozzles N1, N2, Shell on A1-560-ZN-003A - Inspection performed on East, West, South, North, Top, Bottom and Extrados from external. - Echo-to-echo UT technique was used. |  |  |  |  |  |  |  |  |  |  |
| Notes: N/A – not applicable. FSH - Full Screen Height |  |  |  |  |  |  |  |  |  |  |

Inspection drawing

N2 N1 Shell -1

| Results, |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S/NO | Tag No | Item description | DIA(Ø), inch | Material | Nominal thickness, mm | East mm | West mm | South mm | North mm | Top mm | Bottom mm |
| 1 | A1-560-ZN-003A | Nozzle N1 | 8 | SA350M LF2 | 12.7 | 13.12 | 12.90 | - | - | 13.11 | 13.00 |
| 2 |  | Nozzle N2 | 8 | SA350M LF2 | 12.7 | 12.70 | 12.02 | - | - | 12.40 | 11.45 |
| 3 |  | Shell 1 | - | SA 516M Gr.485 N | 16 | 16.00 | 16.40 | 16.50 | 15.46 | - | - |

Inspection drawing

3 2 1

KE01-A1-5600-BU-P-DI-0001-004 / A1-5600-AG-002-8”-A11

| Results, |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location | Line No | Item description | DIA(Ø), inch | Material | Nominal thickness, mm | East mm | West mm | South mm | North mm | Intrados mm | Extrados mm |
| 1 | A1-5600-AG-002-8”-A11 | Elbow | 6 | A333 Gr.6 | 7.11 | 6.61 | 6.92 | - | - | 5.52 | 6.96 |
| 2 |  | Pipe | 6 | A333 Gr.6 | 7.11 | 7.00 | 7.05 | 7.00 | 6.54 | - | - |
| 3 |  | Elbow | 6 | A333 Gr.6 | 7.11 | - | - | 5.46 | 4.50 | 6.45 | 4.80 |

Inspection drawing

1

A1-5600-GA-007-6-A11

| Results, |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location | Line No | Item description | DIA(Ø), inch | Material | Nominal thickness, mm | East mm | West mm | South mm | North mm | Top mm | Bottom mm |
| 1 | A1-5600-AG-007-6”-A11 | Pipe | 6 | A333 Gr.6 | 7.11 | 6.61 | 6.64 | - | - | 6.43 | 7.02 |

| Examined by UT L II cert. No 356 expiry date: 28.04.2021 | Name: Dosbatyrov Damir | Signature:............................... | Date: 12 Oct 2019 |
| --- | --- | --- | --- |
| Approved by UT Level II cer. No 1A 253/15 expiry date: 31.05.2020 | Name: Shakunou Andrei | Signature: ............................ | Date: 12 Oct 2019 |
| Client Representative: | Name: …………………………… | Signature: .............................. | Date: ……………………… |