| ULTRASONIC THICKNESS MEASUREMENT REPORT |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Company/Client |  | NCOC N. V. |  | Project |  | Eskene West |  | Location |  | Gas 2 Tr 2 |
| Line No/Tag No: |  | A1-330-VJ-202 Amin Regenerator |  | Material |  | According to table |  | Día × Nominal Thickness |  | According to table |
| Procedure No: |  | QP-11-UTT-Q01 Rev 03 GOST 28702-90 |  | Calibration Block |  | CS Step wedge |  | Probe S/N |  | D790-SM 1196661 |
|  |  |  |  | Calibration block Range |  | 6.5-25.0 mm |  | Angle |  | 0º |
| Manufacturing Code |  | ASME Sec VIII D1 |  | Calibration Temp |  | +6º |  | Frequency |  | 5 MHz |
|  |  |  |  | Object Temp |  | +104º |  | Size |  | 11 mm |
| Acceptance Standard: |  | Acc. to the client specif. |  | Correction factors |  | Calculated by 38DL in high temperature mode automatically |  | Crystal |  | Dual |
| Surface condition |  | Coated |  | Couplant |  | High temperature UT Gel ʺHiTempco" |  | Accuracy |  | ± 0,1 mm |
| S. No | Equipment/ Material Name |  | Manufacturer |  | Serial No |  | Calibration certificate No |  | Calibration Expiry date |  |
|  | DL-38 |  | OLYMPUS |  | 171421601 |  | BK-01-0674 |  | 20-03-2020 |  |
|  | Step wedge calibration block |  | OLYMPUS |  | 145414 |  | - |  | - |  |
| Job description: - UT measurements were conducted on the Amin Regenerator A1-330-VJ-202 - Echo-to-echo UT technique was used. |  |  |  |  |  |  |  |  |  |  |
| Notes: N/A – not applicable. FSH - Full Screen Height |  |  |  |  |  |  |  |  |  |  |

Drawing

A1-330-VJ-202

3D drawing

Loc R Loc T

| Inspection photos |  |
| --- | --- |
|  | Plate Shell |
| Loc R |  |
|  | Shell Plate Top 123 113 103 9 8 7 6 5 4 3 2 1 63 163 153 143 133 123 113 103 93 83 73 53 43 3 2 1 260 mm 600 mm South Bottom North |
| Loc T |  |
| 1 730 mm X 250 mm spot check - plate size |  |

| Inspection results, mm |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S/N | Item description | Spot location | Nominal thickness shell, mm | Shell, mm |  | Nominal thickness plate , mm |  | Plate , mm |
| 1 | Loc R | Spot-1 | *20+3 | 20.58 |  | No information |  | 19.66 |
| 2 |  | Spot-2 |  | 20.61 |  |  |  | 19.65 |
| 3 |  | Spot-3 |  | 20.60 |  |  |  | 19.66 |
| 4 |  | Spot-4 |  | 20.64 |  |  |  | 19.63 |
| 5 |  | Spot-5 |  | 20.64 |  |  |  | 19.69 |
| 6 |  | Spot-6 |  | 20.60 |  |  |  | 19.65 |
| 7 |  | Spot-7 |  | 20.61 |  |  |  | 19.64 |
| 8 |  | Spot-8 |  | 20.61 |  |  |  | 19.67 |
| 9 |  | Spot-9 |  | 20.63 |  |  |  | 19.66 |
| 10 |  | Spot-10 |  | 20.64 |  |  |  | 19.69 |
| 11 |  | Spot-11 |  | 20.57 |  |  |  | 19.66 |
| 12 |  | Spot-12 |  | 20.58 |  |  |  | 19.67 |
| 13 |  | Spot-13 |  | 20.60 |  |  |  | - |
| 14 |  | Spot-14 |  | 20.62 |  |  |  | - |
| 15 |  | Spot-15 |  | 20.63 |  |  |  | - |
| 16 |  | Spot-16 |  | 20.60 |  |  |  | - |
| 17 | Loc T | Spot-1 | *20+3 | 20.71 |  | No information |  | 19.59 |
| 18 |  | Spot-2 |  | 20.73 |  |  |  | 19.60 |
| 19 |  | Spot-3 |  | 20.69 |  |  |  | 19.60 |
| 20 |  | Spot-4 |  | 20.69 |  |  |  | 19.58 |
| 21 |  | Spot-5 |  | 20.67 |  |  |  | 19.60 |
| 22 |  | Spot-6 |  | 20.72 |  |  |  | 19.63 |
| 23 |  | Spot-7 |  | 20.70 |  |  |  | 19.63 |
| 24 |  | Spot-8 |  | 20.68 |  |  |  | 19.61 |
| 25 |  | Spot-9 |  | 20.72 |  |  |  | 19.64 |
| 26 |  | Spot-10 |  | 20.74 |  |  |  | 19.59 |
| 27 |  | Spot-11 |  | 20.73 |  |  |  | 19.60 |
| 28 |  | Spot-12 |  | 20.73 |  |  |  | 19.61 |
| 29 |  | Spot-13 |  | 20.68 |  |  |  | - |
| 30 |  | Spot-14 |  | 20.69 |  |  |  | - |
| 31 |  | Spot-15 |  | 20.75 |  |  |  | - |
| 32 |  | Spot-16 |  | 20.70 |  |  |  | - |
| Notes: *Nominal thickness is taken according to Technical passport KE01-A1-PPFM001-L27-0007-000 |  |  |  |  |  |  |  |  |
| Examined by UT L II cert. No 628 expiry date: 19.12.2022 |  | Name: Dariskaliyev Nurdaulet |  |  | - Signature: ...................... |  | - Date: 12 November 2019 |  |
| Approved by UT L III cert. No 2B 189/16 expiry date 31.05.2021 |  | Name: Klindukhou Viachaslau |  |  | - Signature: ...................... |  | - Date: 12 November 2019 |  |
| Client Representative: |  | ………………………….…. |  |  | Signature: ...................... |  | Date: .................. |  |