| - PHASED ARRAY ULTRASONIC TESTING REPORT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - JOB DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Client: |  | - NCOC N.V. |  | - Project: | - ESKENE WEST |  |  |  |  |  | - Work Location: |  |  |  | - Gas 2 Tranch-2 |  |
| - JOB DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Brief Description of Job: |  |  |  | - Encoded Thickness Measurement Survey of 28” Elbow |  |  |  |  |  |  |  |  |  |  |  |  |
| - Line No.: |  |  |  | - A1-3002-RG-041-28”-C11-HC |  |  |  | - Location: |  |  |  | - Gas 2 Tranch-2 Unit 300 |  |  |  |  |
| - Material: |  |  |  | - A420 Gr.WPL6 |  |  |  | - Surface Condition: |  |  |  | - Coated |  |  |  |  |
| - Nominal thickness |  |  |  | - 32,62 mm |  |  |  | - Diameter |  |  |  | - 28 inch |  |  |  |  |
| - Part temperature |  |  |  | - 50°C |  |  |  |  |  |  |  |  |  |  |  |  |
| - INSPECTION PROCEDURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Procedure No: |  |  | - QP-11-PAUT-CM-Q01 REV 03 |  |  | - In accordance with: |  |  | - ASME sec V |  |  |  | - In accordance with: |  |  | - Client Specification |
| - INSPECTION EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| S. No | Equipment/ Material Name |  |  | Manufacturer |  |  | Serial No |  |  | Calibration certificate No |  |  |  | Calibration Expiry date |  |  |
|  | OmniScan MX2 |  |  | Olympus |  |  | 103488 |  |  | BK-01-2826 |  |  |  | 26.12.2020 |  |  |
|  | Step wedge calibration block 5-10-20-40 mm |  |  | Fizpribor |  |  | 3255-20 |  |  | C285-20 |  |  |  | 09.06.2021 |  |  |

| - EQIPMENT PARAMETERS |  |  |  |  |  | - CALIBRATION BLOCK DETAILS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mode | - Tx/Rx | Filter | BP 8 MHz | Points quantity | - 640 | Cal block | Step wedge |
| Frequency | - 7.5 MHz | Rectifier | FW | No of elements | - 64 | Material | CS |
| Energy | - 40 V | Video filter | On | Element pitch | 1 mm | Range | 10.0-40.0 mm |
| Pulse width | - 100 ns | Averaging | 1 | Ref sensitivity | 10 dB | Temperature | 29°C |
| PRF | - Auto | Focus depth | - Unfocused | Scan sensitivity | +0 dB | Correction | - n/a |
| Probe | - Olympus Hydroform | Wedge | - n/a | Couplant | Water | Accuracy | - ±0.1 mm |

| - SCAN PLAN |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Test Ref | - Scan type | - Beam type | - Index offset | - Start element | - Active elements | - First Element | - Last Element | - Element Step |
|  | Linear | Compression | 30.5 | 1 | 64 | 1 | 64 | 1 |

| CALIBRATION DETAILS |  |  |  |
| --- | --- | --- | --- |
| Calibration on (10.0-20.0-40.0) mm step wedge block: |  |  |  |
| - DETAILS AND RESULTS |  |  |  |
| - Phased Array inspection was carried out on A1-3002-RG-041-28”-C11-HC elbow for the area of 3-6-9 0’çlock positions with respect to flow. All areas were scanned in increments of 50mm giving an overlap of approx. 11mm and Varied in length and shape to maximise the area covered around the restrictions. Datum points were clearly marked with permanent marker to ensure accurate repeatability. The surface condition was good with Minimal loss of Data due to paint peel off on the surface. - Data collected with (0-350) mm on X-axis, (0-2000) mm on Y-axis. Datum 0 in scan axis started from long seam at 6 O’clock position.950-1050 was not scanned due to the obstruction of UT MAT. |  |  |  |
| INSPECTION DRAWING |  |  |  |
| INSPECTION PHOTO |  |  |  |
| - DETAILS AND RESULTS |  |  |  |
| - A1-3002-RG-041-28”-C11-HC |  |  |  |
|  |  | - Data collected with (0-350) mm on X-axis, (0-2000) mm on Y-axis. Datum 0 in scan axis started from long seam at 6 O’clock position. 950-1050 was not scanned due to the obstruction of UT MAT. |  |
| - The minimum thickness of the A1-3002-RG-041-28”-C11-HC - Elbow |  |  |  |
| - FULL SCAN VIEW OF A1-3002-RG-041-28”-C11-HC - Elbow |  |  |  |
| - RESULTS (A1-3002-RG-041-28”-C11-HC – Elbow) |  |  |  |
| S. No; Inspection Date; Min (mm); X (mm); Y (mm); Average (mm). 1; 04-09-2017; 34.9*; 160-275; 760-950; 36.05. 2; 08-12-2017; 34.54*; 160-320; 760-950; 35.31. 3; 17-03-2018; 34.43*; 160-320; 760-950; 35.31. 4; 26-09-2018; 34.35; 160-320; 760-950; 35.30. 5; 16-07-2019; 34.35; 160-340; 760-950; 35.30. 6; 15-08-2020; 34.36; 160-340; 760-950; 35.30. *Minimum thickness observed at random spots in the mentioned area |
| Examined by - PAUT Level II cert. No 1A 002/18 | Name: Shafar Andrei - expiry date: 30.11.2022 | - Signature: .......................... | - Date: 17 August 2020 |
| Approved by PAUT Level II cert. No 1A 164/16 | Name: Dzmitry Kuryanau expiry date: 28.02.2021 | Signature: .......................... | Date: 17 August 2020 |
| Client Representative: | ………………………….…. | Signature: ..................... | Date: .................. |